CONTROL/INSTALL BAG


Brunton


CLIE


Spare CLIE


Voltmeter 

 
spare 9V radio battery for voltmeter


GPS


Log Sheets 


Manuals (sensor sheets)


bubble level


Cable Ties


Sharpies


Plastic Ruler


HCU and Cable

             direct power cable for the 3T


FUSES

        

10A automotive 

        

Guralp BOB fuses (1A) .

Tools Bag

Portable drill (110V) & charger  

Screwdriver bits for drill  (Phillips, flat)

standard drill bit set AND a 1/2" drill bit

set of Screwdrivers flat and Phillips:


large flat            short wide flat
small flat    micro flat


large Phillips      small Phillips       micro Phillips

set of pliers:

           needle nose        channel locks        robo grips

wrenches:

           wrench set of at least a variable wrench (spanner)

side cutter

small (12") square  - because you should always have one

wire strippers/crimper

chisel

cement trowel

digging (garden) trowel

hammer

wire brush

measuring tape

utility knife 

wood saw (there is a small toolbox sized one by "shark")

hack saw  with spare hacksaw blades

filer   general purpose   round on one side flat on the other

small pry bar /flat bar 

machete

caulking gun (for Si sealant

Flashlight

Construction/other Materials

rolls of self vulcanizing tape(splicing)   or Si tape   - for connectors on the clock and sensors

wire staples (for the PV cable along the PV frame) (use about 10 per site)

cable ties   - for the fence   (use about  30 per site)

for the PV mount:

   7  X 2"x4" (8' length)

   1 4"   3/8" bolt - with two flat washers and a lock washer

   4" nails (common)   or dry-wall screws (4")

machine screws for the PV mount (size not determined yet)

plastic cement mixing tub

water carrying jug

cement mixer hoe

shovel (personal decision)  possibly a pick and a pry bar

post hole digger

fence post hammer

fencing      50 ft plastic (or other) fencing

six fence posts

Silicone sealant (1/2 tube or less per station)

sand   1 bag (50lb)  Quickcrete play sand

           2 bags   (option to put sand on the outside of the vault for better drainage

grout  1/3 bag (50lb)  Quickcrete non-shrinking precision grout

cement  3 bags (40lb)   Quickcrete concrete mix   bottom of the hole 

